A clear vision of our project :

Sense the ground today, predict the risk tomorrow, and protect lives before disaster/accident strikes.

Our problem statement is that - A surface-based wireless sensor network for real-time monitoring of surface deformation and other indicators associated with underground coal mining.

Our goal is not just to detect ground movement, but to detect the warning signs early enough to support safer mines, protect surrounding communities, and move towards smarter and more sustainable mining. Sense the Ground. Predict the Risk. Protect the Future.

To prevent underground mine accident, our system can monitor the effects of underground mining from the surface. It does not directly sense the underground coal seam; instead, it continuously measures surface deformation, tilt, vibration and crack-related changes produced by underground mining. By combining these measurements through a wireless sensor network and AI-based analysis, the system can identify abnormal trends and provide an early indication of possible subsidence. our sensors do not directly measure what is happening underground. They measure surface changes caused by underground mining.

Causes related to underground mining

1. Coal extraction

Coal is removed from underground seams, creating empty spaces (voids) underground. The surrounding rock has to carry the load that was previously supported by the coal.

2. Collapse of mine roof

If the roof rocks above the mined area become unstable, they can deform or collapse. This deformation can gradually move upward toward the surface.

3. Overburden movement

The layers of soil and rock above the coal seam are called overburden. When underground support is reduced, these layers can settle or deform.

4. Pillar failure

In some underground mines, coal pillars are left to support the roof. If pillars become overloaded, damaged, or fail, larger-scale ground movement can occur.

5. Longwall mining

Longwall mining removes a large continuous section of coal. The roof behind the advancing mining face is intentionally allowed to collapse in a controlled manner, which can cause surface subsidence above the panel.

6. Water and groundwater changes

Mining can change groundwater conditions and pore pressure. Water-related weakening or settlement can contribute to ground deformation in some locations.

7. Geological conditions

Faults, weak rock layers, fractures, depth of the coal seam, and thickness of the overlying strata all influence how much ground movement occurs.

8. Abandoned mine workings

Old underground workings may deteriorate over time. Their collapse can sometimes cause delayed ground movement at the surface.

Simple chain for our project

Coal extraction
↓
Underground void created
↓
Rock/roof deformation
↓
Overburden movement
↓
Surface deformation
↓
Tilt + displacement + cracks + vibration
↓
Surface sensor nodes detect changes
↓
AI analyzes the data
↓
Early warning

Underground coal mining removes material that naturally supports the overlying rock strata,the resulting deformation and movement can propagate upward and produce surface subsidence, which our surface sensor network aims to monitor in real time.

Here we use different types of sensors. Those sensors detect various kind of ground movement

The sensors are-     tilt/inclination sensors, vibration sensors, displacement/stretch sensors, crack detection sensors,

Tilt sensor: MPU6050 MEMS Accelerometer + Gyroscope

 Measures small changes in ground inclination.

 Useful for detecting ground tilting before or during subsidence

A tilt sensor measures the angle or inclination of an object relative to a reference direction, usually gravity. In our mine-subsidence project, it is one of the most important sensors because ground movement can cause a sensor node to tilt or change its angle.

For our prototype, we are using the MPU6050.

1. What is MPU6050?

The MPU6050 is a small 6-axis motion sensor containing:

- 3-axis accelerometer → measures acceleration along X, Y and Z axes.

- 3-axis gyroscope → measures rotational/angular velocity around X, Y and Z axes.

It communicates with our ESP32 DevKit using the I²C interface.

The MPU6050 is small, inexpensive and easy to interface with an ESP32, making it suitable for a low-cost prototype. Multiple sensor nodes can be distributed over the monitored area, allowing  to compare changes between locations.

The MPU6050 continuously monitors changes in ground inclination and detects abnormal tilting caused by ground deformation, allowing the system to identify early signs of possible subsidence.

For our prototype, we can monitor:

| Parameter | Purpose |
| --- | --- |

| X-axis acceleration | Detect movement/changes along X |
| --- | --- |

| Y-axis acceleration | Detect movement/changes along Y |
| --- | --- |

| Z-axis acceleration | Reference gravity/movement |
| --- | --- |

| X/Y/Z angular velocity | Detect rotational movement |
| --- | --- |

| Roll angle | Detect side-to-side inclination |
| --- | --- |

| Pitch angle | Detect forward/backward inclination |
| --- | --- |

Vibration sensor:

In our project, the ADXL345 is used as the vibration sensor. It is a 3-axis digital accelerometer that measures changes in acceleration and vibration along the X, Y, and Z axes.

A vibration sensor detects repeated or sudden mechanical movement of an object or structure.

In a mining environment, vibrations can be produced by:

- Movement of heavy mining equipment

- Blasting

- Rock movement

- Roof movement

- Ground deformation

- Sudden structural disturbances

- Machinery operation

The sensor converts these physical movements into electrical/digital measurements that the ESP32 can process.

What is ADXL345?

The ADXL345 is a small, low-power 3-axis accelerometer.

It measures acceleration in:

- X-axis → horizontal movement

- Y-axis → horizontal movement

- Z-axis → vertical movement

It can measure both relatively slow changes and faster vibrations, depending on how it is configured.

A typical ADXL345 module supports selectable measurement ranges such as:

±2 g, ±4 g, ±8 g and ±16 g

where 1 g ≈ 9.81 m/s².

inside the ADXL345 is a microscopic mechanical structure that responds when the sensor accelerates or vibrates. When the ground or structure vibrates, the internal sensing element moves slightly. The ADXL345 converts that movement into acceleration data.

The ESP32 can continuously read acceleration values and look for unusual changes compared with the normal vibration level.

The exact values depend on sensor orientation, mounting, sampling rate and the source of vibration.

Why use 3 axes?

Using three axes is important because ground movement doesn't necessarily happen in only one direction.

The sensor can therefore detect movement in multiple directions.

For our project:

- X-axis: horizontal vibration

- Y-axis: horizontal vibration

- Z-axis: vertical vibration

This gives more useful information than a simple one-direction vibration detector.

Vibration vs tilt — important difference

we are using both MPU6050 and ADXL345, so our presentation should clearly explain why both are included.

| Feature | MPU6050 | ADXL345 |
| --- | --- | --- |
| Main purpose in your system | Tilt/inclination | Vibration/acceleration |
| Accelerometer | Yes | Yes |
| Gyroscope | Yes | No |
| Axes | 3-axis acceleration + 3-axis gyro | 3-axis acceleration |
| Detects vibration | Yes, possible | Yes, primary use here |
| Detects tilt | Yes | Yes, using gravity |
| Power consumption | Low | Very low |
| Interface | I²C | I²C/SPI |

So, in our prototype:

MPU6050 → primarily monitors ground inclination/tilt.

ADXL345 → primarily monitors vibration and acceleration.

This separation makes our system easier to explain.

For our surface mine-subsidence monitoring prototype, the ADXL345 should be mounted firmly inside the sensor node enclosure so that the sensor has a consistent mechanical connection with the node.

Displacement Sensor

In our proposed system, the VL53L0X is being used as the displacement sensor. It is a small ToF (Time-of-Flight) distance sensor that measures the distance between the sensor and a target.

1. What is a displacement sensor?

A displacement sensor measures how far an object has moved from its original position.

For your mine-subsidence project, this is important because ground deformation can cause vertical or horizontal movement.

2. What is VL53L0X?

The VL53L0X is a compact optical distance sensor based on Time-of-Flight technology.

Instead of using an ultrasonic wave, it emits a small infrared laser/light signal toward a target and measures how long the light takes to return.

Crack Sensor —Our crack detection unit consists of a strain or crack gauge connected to an HX711 signal-conditioning module. When ground or a structure deforms, the strain gauge produces a small resistance change. The HX711 amplifies and converts this signal into digital data, which is processed by the ESP32 and transmitted through LoRa. The crack/deformation data is then combined with tilt, vibration and displacement measurements to improve the detection of abnormal ground behavior.

Crack sensor system → detects deformation/crack-related changes; HX711 → amplifies and digitizes the tiny sensing signal.

| Component | HX711 + strain/crack gauge |
| --- | --- |
| Main purpose | Crack/deformation monitoring |
| HX711 type | 24-bit ADC + amplifier |
| Sensing element | Strain gauge / crack gauge |
| Output | Digital data |
| Controller | ESP32 DevKit |
| Interface | Digital interface to ESP32 |
| Application | Strain, deformation, crack monitoring |
| Power | Low |
| Suitable for prototype | Yes |
| Challenge | Proposed solution |
| Sensor failure | Node-health monitoring |
| Battery depletion | Solar + low-power operation |
| LoRa communication loss | Mesh/multi-hop communication |
| Internet failure | Local data storage |
| False alarms | Multi-sensor correlation |
| Limited AI training data | Experimental + simulated dataset |
| Sensor noise | Filtering and calibration |
| Large deployment cost | Low-cost modular nodes |
| Harsh environment | Rugged waterproof enclosure |
| Excessive power consumption | Sleep mode + adaptive sampling |

Important limitation

This is very important for your PPT/viva:

HX711 is not itself a crack sensor.

It is the signal-conditioning and ADC module.

Your complete crack sensing system should be:

Crack/strain sensing element + HX711 + ESP32

Also, a simple load cell is primarily designed to measure force/load, not crack width. If you want to measure actual crack opening, a dedicated crack-width/displacement gauge or suitable strain-based crack sensor is more appropriate.

For a real mine deployment, the sensor should also be ruggedized and calibrated for outdoor conditions.

ESP32 DevKit

The ESP32 DevKit is the main controller (brain) of oour sensor node. It collects data from all the sensors, processes the readings, and sends the information to the LoRa communication module.

1. What is ESP32 DevKit?

The ESP32 DevKit is a development board based on the ESP32 microcontroller.

It provides:

- Digital input/output pins

- Analog input capability

- I²C communication

- SPI communication

- UART communication

- Wi-Fi

- Bluetooth/BLE

- Low-power operating modes

For our project, you mainly use it to read sensors, process data, and communicate with LoRa.

SENSOR NODE

│

┌───────────┼────────────┐

↓           ↓            ↓

MPU6050    ADXL345     VL53L0X

Tilt      Vibration   Displacement

│           │            │

└───────────┼────────────┘

↓

HX711 + Gauge

Crack/Strain

│

↓

ESP32 DevKit

"Main Controller"

↓

LoRa SX1278

Sensors

↓

ESP32

↓

Data processing

↓

Add Node ID + Time + Sensor values

↓

LoRa

The ESP32 DevKit acts as the central processing unit of each sensor node. It collects data from the MPU6050 tilt sensor, ADXL345 vibration sensor, VL53L0X displacement sensor and HX711-based crack/strain sensing unit. It processes and organizes the sensor data before sending it through the SX1278 LoRa module to the gateway. The received data is then analyzed on the server and displayed on the web dashboard for real-time monitoring and early warning.

Power Supply and Protection System for our Mine Sensor Node

For our mine-subsidence monitoring prototype, these components provide continuous and protected power to the ESP32, sensors and LoRa module:

Solar Panel → Solar Charge Controller → Battery → Boost Converter → Electronics

And everything is housed inside a rugged waterproof enclosure.

1. 🔋 Battery

The battery stores electrical energy generated by the solar panel so the sensor node can operate:

- At night

- During cloudy weather

- When solar generation is temporarily low

For our prototype, a rechargeable Li-ion  battery can be considered.

Main function: Energy storage.

Solar energy

↓

Charge controller

↓

Battery

↓

ESP32 + Sensors + LoRa

2. ☀️ Solar Panel

The solar panel converts sunlight into electrical energy.

☀ ☀ ☀

SUNLIGHT

↓

┌───────────┐

│   SOLAR   │

│   PANEL   │

└─────┬─────┘

↓

Electrical power

It supplies power to the charging system and ultimately keeps the battery charged.

Main function: Energy generation.

For our prototype, the panel should be sized according to:

- ESP32 consumption

- Sensor consumption

- LoRa transmission frequency

- Battery capacity

- Number of sunlight hours

- Expected cloudy/rainy conditions

### 3. 🔌 Solar Charge Controller

- The charge controller is placed between the solar panel and battery.

Its job is to manage the charging process and help protect the battery from inappropriate charging conditions.

Depending on the battery chemistry, you need a controller specifically compatible with that battery.

Main function: Safe and controlled battery charging.

### 4. ⚡ Boost Converter

A boost converter increases a lower DC voltage to a higher required voltage.

Battery

↓

Low DC voltage

↓

Boost Converter

↓

Required voltage

↓

ESP32

However, whether you actually need a boost converter depends on your selected battery and the voltage requirements of your electronics.

For example, if your battery voltage cannot directly provide the required regulated voltage, a suitable DC-DC converter can be used.

Main function: Convert battery voltage to the required stable voltage.

5. 🛡️ Rugged Waterproof Enclosure

This is particularly important for your project because the sensor nodes are intended for outdoor deployment.

The enclosure protects:

- ESP32

- LoRa module

- Battery

- Charge controller

- Boost/DC-DC converter

- HX711

- Wiring and connectors

from environmental conditions such as:

- Rain

- Dust

- Moisture

- Mud

- Mechanical impact

- Outdoor exposure

A suitable enclosure should have an appropriate IP rating, robust cable glands, secure mounting and proper sealing.

Important: Waterproofing the enclosure can also affect heat dissipation, so the electronics should not simply be sealed without considering thermal management.

☀ SUNLIGHT

↓

SOLAR PANEL

↓

SOLAR CHARGE

CONTROLLER

↓

BATTERY

↓

BOOST / DC-DC

CONVERTER

ESP32 +

SENSORS

+ LoRa

And the whole system is installed inside:

│     RUGGED WATERPROOF ENCLOSURE

│

│  Battery

│  Charge Controller

│  DC-DC Converter

│  ESP32

│  Sensors

│  LoRa

The sensor node is powered by a solar energy system consisting of a solar panel, solar charge controller, rechargeable battery and regulated DC-DC converter. The solar panel generates electrical energy, the charge controller manages battery charging, and the battery provides power when sunlight is unavailable. A suitable converter provides the required regulated voltage to the ESP32, sensors and LoRa module. All electronic components are protected inside a rugged, waterproof enclosure suitable for outdoor monitoring.

Solar panel = generates power | Charge controller = manages charging | Battery = stores power | DC-DC converter = provides required voltage | Rugged enclosure = protects the electronics.

☀️

SOLAR PANEL

↓

CHARGE CONTROLLER

↓

BATTERY

POWER MANAGEMENT

↓

ESP32 DevKit

MPU6050        → Tilt

ADXL345        → Vibration

VL53L0X        → Displacement

HX711 + Crack   → Crack

sensing

│

SX1278 LoRa

│

ANTENNA

│

▼

LoRa Network